# EDA DCI Lab — Technical Node Documentation

Datacenter interconnect test lab on Talos EDA with Containerlab fabric srl-leaf-spine-dcgw. Documents fabric nodes, virtual networks, client attachments, DCI BGP policies, and cross-DC verification tests.

# 1. Environment

| Item | Value |
| --- | --- |
| EDA UI | https://100.124.186.55 |
| SSH / kubectl | nokia@100.124.186.51 |
| Namespace | clab-srl-leaf-spine-dcgw |
| CLAB name | srl-leaf-spine-dcgw |
| Mgmt subnet | 172.65.10.0/24 |

# 2. Architecture summary

Two site fabrics (DC1 pod-1 ASN 65401, DC2 pod-2 ASN 65402) connect via four DCGW nodes with MPLS-based DCI. L3 services use RouterInterconnect (EVPN-VXLAN on leaves, IPVPN-MPLS on DCGW). L2 services use BridgeDomainInterconnect (EVPN-MPLS on DCGW). WAN PE routers attach via backbone-simulation fabric.

# 3. Fabric and WAN nodes

| Node | Site | Role | Platform | Mgmt IPv4 | Fabric |
| --- | --- | --- | --- | --- | --- |
| srl-leaf-1 | DC1 | leaf | ixr-d2l | 172.65.10.100 | pod-1 |
| srl-leaf-2 | DC1 | leaf | ixr-d2l | 172.65.10.101 | pod-1 |
| srl-leaf-3 | DC1 | leaf | ixr-d3l | 172.65.10.102 | pod-1 |
| srl-leaf-4 | DC1 | leaf | ixr-d3l | 172.65.10.103 | pod-1 |
| srl-spine-1 | DC1 | spine | ixr-d4 | 172.65.10.104 | pod-1 |
| srl-spine-2 | DC1 | spine | ixr-d4 | 172.65.10.105 | pod-1 |
| srl-leaf-5 | DC2 | leaf | ixr-d2l | 172.65.10.106 | pod-2 |
| srl-leaf-6 | DC2 | leaf | ixr-d2l | 172.65.10.107 | pod-2 |
| srl-leaf-7 | DC2 | leaf | ixr-d3l | 172.65.10.108 | pod-2 |
| srl-leaf-8 | DC2 | leaf | ixr-d3l | 172.65.10.109 | pod-2 |
| srl-spine-3 | DC2 | spine | ixr-d4 | 172.65.10.110 | pod-2 |
| srl-spine-4 | DC2 | spine | ixr-d4 | 172.65.10.111 | pod-2 |
| dcgw-1 | DC1 | dcgw | ixr-x1b | 172.65.10.200 | pod-1 |
| dcgw-2 | DC1 | dcgw | ixr-x1b | 172.65.10.201 | pod-1 |
| dcgw-3 | DC2 | dcgw | ixr-x1b | 172.65.10.202 | pod-2 |
| dcgw-4 | DC2 | dcgw | ixr-x1b | 172.65.10.203 | pod-2 |
| sros-pe-1 | WAN | pe | sr-1 | 172.65.10.204 | backbone |
| sros-pe-2 | WAN | pe | sr-1 | 172.65.10.205 | backbone |

# 4. Virtual networks and DCI

| VNet | Tier | BD | Subnet | RT | RD/EVI | Interconnect CR | DCGW CP | Stretch |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vnet-1 | L3 IRB | bd-1 | 172.16.101.0/24 | target:1:100 | 65401:2 | RouterInterconnect | IPVPN-MPLS | DC1 native → DC2 |
| vnet-2 | L3 IRB | bd-2 | 172.16.201.0/24 | target:1:103 | 65402:2 | RouterInterconnect | IPVPN-MPLS | DC2 native → DC1 |
| vnet-3 | L2 BD | bd-3 | 172.16.103.0/24 | target:1:104 | EVI 104 / 110 | BridgeDomainInterconnect | EVPN-MPLS | DC1 native → DC2 |
| vnet-4 | L2 BD | bd-4 | 172.16.105.0/24 | target:1:105 | EVI 105 / 111 | BridgeDomainInterconnect | EVPN-MPLS | DC2 native → DC1 |

# 5. Client attachment matrix

Each vnet uses a dedicated test subnet. Cross-DC tests pair the same vnet on both sites (same broadcast domain / IRB prefix via stitch).

| Client | Site | Service IP | VNet | Leg | Leaf / port | Cross-DC peer |
| --- | --- | --- | --- | --- | --- | --- |
| client-vnet-1-dc1 | DC1 | 172.16.101.1 | vnet-1 | L3 native | leaf-1 e1-5 | client-vnet-1-dc2 |
| client-vnet-1-dc2 | DC2 | 172.16.101.2 | vnet-1 | L3 stretched | leaf-8 e1-6 | client-vnet-1-dc1 |
| client-vnet-2-dc2 | DC2 | 172.16.201.1 | vnet-2 | L3 native | leaf-5 e1-5 | client-vnet-2-dc1 |
| client-vnet-2-dc1 | DC1 | 172.16.201.2 | vnet-2 | L3 stretched | leaf-3 e1-5 | client-vnet-2-dc2 |
| client-vnet-3-dc1 | DC1 | 172.16.103.1 | vnet-3 | L2 native | leaf-2 e1-6 | client-vnet-3-dc2 |
| client-vnet-3-dc2 | DC2 | 172.16.103.2 | vnet-3 | L2 stretched | leaf-6 e1-6 | client-vnet-3-dc1 |
| client-vnet-4-dc2 | DC2 | 172.16.105.1 | vnet-4 | L2 native | leaf-5 e1-6 | client-vnet-4-dc1 |
| client-vnet-4-dc1 | DC1 | 172.16.105.2 | vnet-4 | L2 stretched | leaf-4 e1-6 | client-vnet-4-dc2 |

# 6. Port conventions

e1-5 — L3 IRB edges (vnet-1 on DC1 leaves, vnet-2 on DC2 leaves).

e1-6 — L2 bridge edges (vnet-3 DC1, vnet-4 DC2) plus stretched L3/L2 legs.

client-vnet-{N}-dc{X} — one single-homed client per vnet per DC (8 total).

# 7. DCI BGP peers and policies

| Peer pair | Link role | Export policy | Import policy |
| --- | --- | --- | --- |
| dcgw-1 ↔ dcgw-3 | Primary | export-wan-routes-only-dc-1 | import-dci-services-dc-1 |
| dcgw-2 ↔ dcgw-4 | Secondary + SOO | export-dc-1-routes-and-add-soo | import-dci-services-dc-1 |
| dcgw-3 ↔ dcgw-1 | Primary | export-wan-routes-only-dc-2 | import-dci-services-dc-2 |
| dcgw-4 ↔ dcgw-2 | Secondary + SOO | export-dc-2-routes-and-add-soo | import-dci-services-dc-2 |

Loop avoidance: DC1 local routes use internal tag-10; DC2 local tag-20. Secondary links add SOO (soo-1122 / soo-2211). DCI peers enable EVPN and VPN-IPv4.

# 8. Verification tests

| Case | Forward ping | Reverse ping |
| --- | --- | --- |
| L3 vnet-1 | client-vnet-1-dc1 → 172.16.101.2 | client-vnet-1-dc2 → 172.16.101.1 |
| L3 vnet-2 | client-vnet-2-dc2 → 172.16.201.2 | client-vnet-2-dc1 → 172.16.201.1 |
| L2 vnet-3 | client-vnet-3-dc1 → 172.16.103.2 | client-vnet-3-dc2 → 172.16.103.1 |
| L2 vnet-4 | client-vnet-4-dc2 → 172.16.105.2 | client-vnet-4-dc1 → 172.16.105.1 |

Run on Talos host: bash eda-dci-lab/scripts/test-cross-dc-ping.sh

# 9. Deployment workflow

1. Copy clab/clab-s-spine-spine-leaf-srl-only.yaml to ~/3-tier-dci and clab deploy.

2. Register topology with EDA (fabric pod-1, pod-2, backbone-simulation).

3. Apply services: bash eda-dci-lab/scripts/apply-all.sh

4. Verify BGP on DCGW DCI peers (Established, EVPN + VPN-IPv4).

5. Run cross-DC ping matrix.

# 10. Related paths

CLAB YAML: eda-dci-lab/clab/clab-s-spine-spine-leaf-srl-only.yaml

EDA bundle: eda-dci-lab/services/ (interconnect, policies, interface labels)

README: eda-dci-lab/README.md